# Assignment 14 – DNS Server Configuration

| Student Name | Shreya Panchal |
| --- | --- |

## Theory

### What is DNS?

DNS (Domain Name System) is a network service that translates domain names into IP addresses. It allows users to access websites using easy-to-remember names instead of numeric IP addresses.

### How DNS Resolves Domain Names

- User enters a domain name (e.g., musicmania.in).

- The DNS Resolver receives the request.

- The DNS Server checks its records.

- The corresponding IP address is returned.

- The client connects to the destination server.

## Practical

### Aim

Configure a DNS Server in Cisco Packet Tracer and verify domain name resolution.

### Devices Used

- 1 Server

- 1 Switch

- 1 PC

### Steps

- Place one Server, one Switch, and one PC in Cisco Packet Tracer.

- Connect all devices using Copper Straight-Through cables.

- Configure the Server:

- IP Address: 192.168.1.50

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- Open Services → DNS and turn DNS ON.

- Add an A Record:

- Name: musicmania.in

- Address: 192.168.1.50

- Configure the PC:

- IP Address: 192.168.1.100

- Subnet Mask: 255.255.255.0

- Gateway: 192.168.1.1

- DNS Server: 192.168.1.50

Open Command Prompt and run:

nslookup musicmania.in

- Verify that the domain resolves successfully.

Result: The DNS server successfully resolved musicmania.in to 192.168.1.50.

## Task 1

Install and configure a DNS Server. Create a DNS zone for musicmania.in.

Answer

A DNS Server was installed and configured successfully. A forward lookup zone named musicmania.in was created, and the DNS service was enabled. The server was assigned the IP address 192.168.1.50.

## Task 2

Add an A Record mapping www.musicmania.in to 192.168.1.50 and verify it using nslookup.

Answer

An A Record was created with the following details:

- Record Name: www.musicmania.in

- IP Address: 192.168.1.50

Verification command:

nslookup www.musicmania.in

Output:

Name: www.musicmania.in Address: 192.168.1.50

Result: The A Record resolved correctly to the configured IP address.

## Task 3

Create a CNAME Record for play.musicmania.in pointing to www.musicmania.in.

Answer

A CNAME Record was created:

- Alias: play.musicmania.in

- Target: www.musicmania.in

Verification command:

nslookup play.musicmania.in

Output:

Name: play.musicmania.in Canonical name: www.musicmania.in Address: 192.168.1.50

Result: The alias successfully redirected to the main domain.

## Task 4

DNS Troubleshooting

Answer

If the DNS server does not resolve the domain, follow these troubleshooting steps:

- Check that the DNS Service is running.

- Verify the DNS zone and A Record configuration.

- Ensure the client is using the correct DNS Server IP (192.168.1.50).

- Restart the DNS service if required.

Flush the DNS cache using:

ipconfig /flushdns

Test name resolution using:

nslookup musicmania.in

Verify network connectivity using:

ping 192.168.1.50

Result: After correcting the DNS configuration and clearing the cache, the domain resolved successfully.

## Reference Diagram

The following diagram illustrates the DNS resolution process described above: