# ASSIGNMENT 11

Build and Configure a Small Office (SOHO) Network

Shreya Panchal

# Task 1: Assign Static IP Addresses

## Theory

A Static IP Address is manually assigned to a device. It remains fixed and is commonly used for servers, printers, and network devices.

## Practical Answer

### IP Address Configuration

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC-1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC-2 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-3 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 |
| Laptop-1 | 192.168.1.13 | 255.255.255.0 | 192.168.1.1 |
| Laptop-2 | 192.168.1.14 | 255.255.255.0 | 192.168.1.1 |

### Verification Command

ping 192.168.1.11

### Result

All five devices were successfully connected and replied to the ping request.

# Task 2: Configure a DHCP Server

## Theory

DHCP (Dynamic Host Configuration Protocol) automatically assigns IP addresses, subnet masks, default gateways, and DNS servers to client devices.

## Practical Answer

### DHCP Configuration

- DHCP Server IP: 192.168.1.1

- IP Address Pool: 192.168.1.100 – 192.168.1.150

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- DNS Server: 8.8.8.8

### Verification

Run:

ipconfig

### Result

| Device | Assigned IP |
| --- | --- |
| PC-1 | 192.168.1.100 |
| PC-2 | 192.168.1.101 |
| Laptop-1 | 192.168.1.102 |

All devices received unique IP addresses from the DHCP server.

# Task 3: Configure a DNS Service

## Theory

DNS (Domain Name System) converts domain names into IP addresses, allowing users to access websites using names instead of IP addresses.

## Practical Answer

### DNS Record

| Hostname | IP Address |
| --- | --- |
| musicplayer.local | 192.168.1.20 |

### Verification Command

nslookup musicplayer.local

### Result

The hostname musicplayer.local successfully resolved to 192.168.1.20, and the local webpage opened correctly.

# Task 4: Create Two VLANs

## Theory

A VLAN (Virtual Local Area Network) divides one physical switch into multiple logical networks to improve security and reduce broadcast traffic.

## Practical Answer

### VLAN Configuration

| VLAN ID | VLAN Name | Devices |
| --- | --- | --- |
| VLAN 10 | Workstations | PC-1, PC-2 |
| VLAN 20 | Smart Devices | Laptop-1, Laptop-2 |

### Connectivity Test

| Test | Result |
| --- | --- |
| PC-1 → PC-2 | Successful |
| Laptop-1 → Laptop-2 | Successful |
| PC-1 → Laptop-1 | Failed |

### Result

Devices in the same VLAN communicated successfully, while communication between different VLANs failed because Inter-VLAN Routing was not configured.

# Task 5: Configure NAT for Internet Access

## Theory

NAT (Network Address Translation) allows private IP addresses to access the internet using one public IP address.

## Practical Answer

### NAT Configuration

- Inside Network: 192.168.1.0/24

- Public IP: 203.0.113.10

### Verification

Command:

ping 8.8.8.8

Command:

ping www.wikipedia.org

### Result

All LAN devices successfully accessed the simulated internet through the router using NAT.

# Overall Network Configuration

| Component | Configuration |
| --- | --- |
| Network | 192.168.1.0/24 |
| Router IP | 192.168.1.1 |
| DHCP Range | 192.168.1.100 – 192.168.1.150 |
| DNS Server | 8.8.8.8 |
| VLAN 10 | Workstations |
| VLAN 20 | Smart Devices |
| NAT | Enabled |

# Troubleshooting Documentation

| Problem | Cause | Solution |
| --- | --- | --- |
| Ping Failed | Wrong IP Address | Configure correct IP address |
| DHCP Not Assigning IP | DHCP Service Disabled | Enable DHCP Server |
| DNS Not Resolving | Wrong DNS Address | Configure correct DNS Server |
| Different VLAN Communication Failed | No Inter-VLAN Routing | Configure Router-on-a-Stick |
| No Internet Access | NAT Not Configured | Enable NAT on Router |

# Conclusion

A Small Office (SOHO) Network was successfully configured using proper IP addressing and subnetting. DHCP automatically assigned IP addresses to client devices, DNS resolved custom hostnames correctly, VLANs were created to segment the network, and NAT enabled internet connectivity. Connectivity was verified using the ping command, and common networking issues were identified and resolved through troubleshooting. This configuration provides a secure, efficient, and fully functional LAN suitable for a small office environment.